5200/R Optical Receiver Features [image: image1.emf]
◇ High quality E-O PIN-diode receiving module 

◇ Four-stage input optical power LED display

 ◇ Wide range of input optical power 

◇ PHILIPS Power Doublers output module 

◇ Plug-in splitter(tap) in output and plug-in return path module

 ◇ Local and remote powering 

◇ Weatherproof and RF-screened die-cast housing 

Description 

WR5200 Series Field Optical Receiver are the most common transmission equipment of CATV HFC, which mainly used in long distance optical fiber transmission for TV video signal, digital TV signal, telephone voice signal and data signal(compressed data). Since our first CATV optical receiver come to market in 1995, we have done deep research on all kinds of technical performance parameters for more than 10years, accumulating a lot of experience in product development and engineer in practice. The internal circuit and external structure are much better and the performance is upgraded. 

	Items
	Unit
	Performance Parameter

	OpticalWavelength/Interface
	nm
	1300～1560nm，SC or FC/APC

	Return Loss
	dB
	≥48（APC face）

	Optical Power Input Range
	dBm
	-9～+2dBm（-3～-4dBm receiving suggested）

	Frequency Range/Flatness
	MHz/dB
	45～1000/±0.75

	Operating fiber type
	
	single mode

	RF Output Level
	dBuV
	1×108 or 2×104，1×104 or 2×102

	EQ Output Noise Current
	PA/
	≤8

	AGC Specs
	dB
	-7～+2dBm/Nominal Output Level±1

	C/N（-1dBm receive）
	dB
	≥52

	CTB/CSO
	dB
	68/70（as 2×104）

	Output Attenuation/EQ
	dB
	0～18（7～9 dB vice EQ suggested）

	RF Port Return Loss
	dB
	≥16（45～862MHz）

	RF Output Connector
	
	FL-10 Metric or Inch

	Operation/Storage Temperature
	℃
	-30～+60/-40～+75

	Power Supply/Consumption
	V/W
	160～260V 35～75V convert 

	EOC Insertion Loss
	dB
	≥1（5-30/65MHz）

	Ability of Anti-lightning
	Kv
	5

	Outline/Installation Size
	mm
	205x140x80mm 

	Relative humidity
	
	max. 95% no condensation


